BOA 2000 EXM2 MONITOR (7451X2)

Product Specification

An Elkhart Brass monitor shall be provided that features an integrated Wi-Fi communication system that allows for configuration and diagnostics via a Wi-Fi capable device. The monitor shall be constructed from durable, hard anodized, lightweight Elk-O-Lite® material with a 4” vaned waterway for flows up to 2000 gpm (8000 lpm) ; shall be configured with a 4” 150# inlet flange and 3.5” NHT male outlet; shall have a stow height of not more than 20.0”  (508 mm) and a swing radius of not more than 8.9” (226 mm); shall have two (2) gear motors that allow for simultaneous vertical and horizontal adjustment; one motor shall control up to 350 degree horizontal rotation; one motor shall control vertical movement up to 165 degrees (-45 degrees to +120 degrees) 30 degrees extended travel capability; horizontal and vertical motors shall have an override device for use in the event of a power failure; electric controls shall be NEMA 4 rated and allow for programmable horizontal center position, vertical and horizontal stops, stow and deploy positions, keep out zones and motors speeds fast or slow; electric control shall allow for horizontal and vertical oscillation; electric control shall be CAN compatible; shall be compatible with both 12VDC and 24VDC power supply; shall include auxiliary output for driving an external device (i.e. light or camera).